Safe Operating Procedure (SOP) BvL Mixer Wagon

1. Purpose

To ensure the safe and efficient operation of the BvL mixer wagon during feed mixing and distribution on the farm.

2. Scope

This SOP applies to all operators and maintenance personnel involved in the use of the BvL mixer wagon.

3. Responsibilities**

- **Operators** must follow this SOP at all times.

- **Supervisors** must ensure all operators are trained and competent.

Safety Precautions**

- Do not enter mixer wagon unless you have shutdown tractor and pulled key out

- Never enter the mixer while it is running.

- Keep bystanders, especially children, away from the machine.

- Do not bypass safety guards or interlocks.

- Always follow the manufacturer’s manual for specific instructions.

- Keep clear of PTO shaft while operating

4. Personal Protective Equipment (PPE)

- Safety boots

- Overalls

- No loose clothing

- Hearing protection (if operating near loud machinery)

- Gloves (when handling feed or performing maintenance)

5. Pre-Operation Checks

- Inspect the mixer wagon for visible damage or wear.

- Check hydraulic hoses and connections for leaks.

- Ensure all guards and shields are in place and secure.

- Verify that the PTO shaft is properly connected and shielded.

- Check oil levels, tire pressure, and general cleanliness.

- Ensure the scale system (if equipped) is functioning correctly.

- Check inside mixer wagon

-  Inspect tires and wheel nuts

6. Operating Procedure**

1. **Start-Up**

- Connect the mixer wagon to the tractor securely.

- Engage the PTO at low RPM and gradually increase to operating speed.

- Ensure the mixer is on level ground before loading.

2. **Loading**

- Load ingredients in the correct order as per the feed ration.

- Do not overload the mixer beyond its rated capacity.

- Keep hands and tools away from the mixing chamber during operation.

- Ensure bystanders are clear of tractor and loading area

3. Mixing

- Allow sufficient time for thorough mixing (follow manufacturer guidelines).

- Monitor the mix visually and via the scale system if available.

-Do not turn while PTO is running

4. Discharge

- Position the mixer wagon at the feeding area.

- Engage the discharge mechanism slowly.

- Ensure no one is near the discharge chute during operation.

🛠 **Maintenance**

- Follow greasing and oiling schedule

- Inspect auger blades regularly

- Monitor gearbox oil levels

- Replace worn parts promptly

Storage

- Clean the mixer wagon thoroughly to prevent feed build-up.

- Inspect for wear or damage and report any issues.

- Store the mixer wagon in a dry, secure location.

.